# Lab 03: ROS Tutorial 1

By Sean Lu, last modified on 09/25 2018.

The objective of this tutorial is to introduce to the ROS concept, nodes, publishers and subscribers, and also demonstrate some commonly used command line tools. Spatial transformation between each component is an important thing in your robot, luckily, tf helps us to do the job. We have assumed you have the basic coding ability to both C++ and Python. You should also understand homogeneous transformation matrix, if not, Prob. Yang’s  Robotics would be a good material for you. You should have installed ROS in your laptop, for those who use native Ubuntu please refer to the Hardware and Software Setup part.

## Hardware and Software Setup

We have already installed ROS in the VirtualBox image we provided, for those who use native Ubuntu, please follow the step here.

Clone the repo and make the workspace

laptop $ cd ~/

laptop $ git clone https://github.com/Sensing-Intelligent-System/sis_lab_all_2020

# You will be asked to type your Github username and password

laptop $ cd ~/sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws

laptop $ catkin_make # This may take awhile

laptop $ source devel/setup.bash # Make ROS knows where your packages are, do this every time you open a new terminal

Download the bags we prepared here

temperature_pressure_bag

[Download in your VirtualBox]

Put the bag in sis_lab_all_2020/03-ROS_tutorial_1/bag

laptop $ cd ~/Downloads

laptop $ mkdir ~/sis_lab_all_2020/03-ROS_tutorial_1/bag && mv *.bag ~/sis_lab_all_2020/03-ROS_tutorial_1/bag/

Lab 03: ROS Tutorial 1	1

Hardware and Software Setup	1

Overview	2

Topics and Activities	2

Topic/Activity 1 Command Line Tools	2

Topic/Activity 2 Publisher/ Subscriber	6

Topic/Activity 3 TF	9

Assignment Tasks	12

Task 1 Publisher/ Subscriber	12

Task 2 TF	13

Reference	15

## Overview

Estimated Time to Finish: 2 hours

After completing this tutorial you should

- understand the basic ROS concept: nodes, publishers and subscribers

- understand how to use tf APIs

- be able to write a simple node to publish/ subscribe the specific topic

## Topics and Activities

### Topic/Activity 1 Command Line Tools

ROS provides many convenient command line tools, let’s get familiar with them.

Topic 1.1 Initialize your ROS master

Open a new terminal, type

laptop $ roscore

You will see something like the following

Open another terminal, type

laptop $ rostopic list

You will see there are two topics, /rosout and /rosout_agg

Topic 1.2 Play the bag

Let’s see what surprise inside the bag,

laptop $ cd ~/sis_lab_all_2020/03-ROS_tutorial_1/bag

laptop $ rosbag info temp_pressure_2018-09-21-01-05-38.bag

As you can see, the information include the duration, start and end time, bag size, total number of messages, the message type and topic name.

laptop $ rosbag play temp_pressure_2018-09-21-01-05-38.bag -l # -l means play in loops

Open another terminal, type

laptop $ rostopic list

You will see two new topics as expected

Info helps us see who publishes and subscribes the topic

laptop $ rostopic info /sensor/temp

/sensor/temp is published by the node play, and no subscriber yet.

If we are interested in the rate of the topic:

laptop $ rostopic hz /sensor/temp

The terminal keeps printing data until you press Ctrl+C.

Echo prints the data of the topic:

laptop $ rostopic echo /sensor/temp

Press Ctrl+C to stop.

We can also visualize the temperature-time curve,

laptop $ rqt_plot

A window with name rqt_plot will show with empty canvas, in Topic field, add /sensor/temp

Then you will see the curve as following,

you can adjust the axes range in the blue box in above picture. Press ‘X’ to turn off the interface.

Topic 1.3 What messages and services I have?

There are many default messages and services in ROS, let’s take a look.

laptop $ rosmsg list

There will be several lines, for prefix ‘s’, you will see a family std_msgs:

These are the most basically message types in ROS, you can find them in /opt/ros/kinetic/share/std_msgs/msg.

Other usually used message family include geometry_msgs, sensor_msgs and visualization_msgs.

If you want to see what information inside the message, for example, sensor_msgs/Image, you can type

laptop $ rosmsg info sensor_msgs/Image

The result is as following

Just like rosmsg,

laptop $ rossrv list

You will see many available services in your system, a screenshot of the result is as below

So what SetBool do?

laptop $ rossrv show std_srvs/SetBool

You call the service with ‘data’, the service will return a response to you whether if it is ‘success’ and a string ‘message’.

Checkpoint 1: Show TA your rqt_plot

### Topic/Activity 2 Publisher/ Subscriber

Now we are going to run a ROS node, open a terminal

laptop $ cd ~/sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws && source devel/setup.bash

laptop $ rosrun tutorial convert_temp_unit.py

The result should be as following

The number after INFO is the time now, you can get similar string in Python if you print time.time(). The string after colon is our information, what degrees Fahrenheit were at that time.

Let’s see what contents in the file, open a new terminal

laptop $ cd ~/sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws && source devel/setup.bash

laptop $ roscd tutorial/src

laptop $ vim convert_temp_unit.py (or $ code convert_temp_unit.py)

In this node, we subscribe to /sensor/temp, convert its unit to Fahrenheit and then publish as topic /sensor/temp_fah, we also print the information as above.

What is our nodes/ topics flow now? Exit vim and type

laptop $ rqt_graph

/play publishes /sensor/temp and /convert_temp_unit_node subscribes to it, the topics with no subscribers will not shown in rqt_graph.

With command

laptop $ rosnode list

we can list all active nodes just like “rostopic list”

How to create an user-defined message if we want to publish temperature and pressure simultaneously? Follow the step here to create the new message type, my_message, with two float64: temp and pressure.

Next,

laptop $ rosrun tutorial test_my_message.py

In another terminal,

laptop $ rostopic echo /sensor/my_msg

You will see

If you want to know what this node has done, see sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws/src/tutorial/src/test_my_message.py

We subscribe to both /sensor/temp and /sensor/pressure, get the data and publish our new type message.

Checkpoint 2: Show TA the picture above

### Topic/Activity 3 TF

Notation: TAB: homogeneous transformation matrix from frame A to frame B

Topic 3.1 Publish a transform

Given TAB= and TBC=, how to build the transform tree like the figure below with the help of TF?

In this tutorial, we use C++ to construct our node, while don’t forget that ‘rosrun’ must be feed with an executable file, so you have to make your C++ source code be executable. In ROS Kinetic, we use CMake and catkin to lighten our burden. Take a look at sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws/src/tutorial/CMakeLists.txt

laptop $ cd ~/sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws && source devel/setup.sh

laptop $ roscd tutorial

laptop $ vim CMakeLists.txt

At Line 155, you will see

For more detail what the function of each tag, please refer to here. Don’t edit the other part of CMakeLists.txt manually unless you know what you are doing. Every time you create a new C++ file, add these three tags to your CMakeLists.txt; or if you have modified your C++ file, back to ~/sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws, and then

laptop $ catkin_make # You can skip this step until Assignment 2

Run the node, stop all your terminal unless roscore and in one terminal

laptop $ rosrun tutorial test_broadcast

And in another terminal,

laptop $ cd ~/sis_lab_all_2020/03-ROS_tutorial_1/rviz

laptop $ rviz -d tf.rviz

You will see something like below

Let’s see the transform tree, in another terminal

laptop $ cd ~/ && rosrun tf view_frames

It will produce an PDF file named frames.pdf, use evince to take a look

laptop $ evince frames.pdf

Just like the structure we expected.

Print the transform between frame A and frame B by

laptop $ rosrun tf tf_echo /A /B

Expected result

Topic 3.2 Listen to the transform

Sometimes we have to know the transformation between two frames, for pick and place scenario, we already know the transformation between the camera and the object, but what we want to know is the one from the base of the robot arm to the object, so that we could pick it with the tool on the arm. TF provides listener API which complete this job, let’s study it. In one terminal,

laptop $ rosrun tutorial test_listener

This node will print the transformation between frame A and frame C in one shot rather than tf_echo. The expected result will be looked like the following

You may take a look at sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws/src/tutorial/src/transform_listener.cpp

Checkpoint 3: Show TA your rviz, and echo the transform from frame A to frame C

## Assignment Tasks

Students should do this by themselves

### Task 1 Publisher/ Subscriber

In this assignment, we ask you to do the similar task with Topic 2, convert temperature from Celsius to Fahrenheit, while this time we want to know the pressure in mmHg rather than in hPa. Edit ~/sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws/src/tutorial/src/assignment1.py

such that you have the result as following

Expected Result:

Fig. 1.1 $ rosrun tutorial assignment1.py

[Hint: Don’t forget to make your .py file executable by

‘$ chmod +x assignment1.py’]

Fig. 1.2 $ rostopic list

Fig. 1.3 $ rostopic echo /sensor/pressure_mmhg

### Task 2 TF

In this assignment, we ask you to do the similar task with Topic 3, while this time with more complicated transform tree as following, where

TBA=, TAC=,

TED=, TCD=

TAF=

Edit ~/sis_lab_all_2020/03-ROS_tutorial_1/catkin_ws/src/tutorial/src/assignment2.cpp such that you have the expected result illustrated below

Expected Result:

Fig. 2.1 $ rviz -d tf.rviz

Fig. 2.2 $ rosrun tf tf_echo /B /F

[Hint: Many tools available here]

## Reference

rospy API document