# Lab09 PCL and ICP

By Michael Su, Lily Hsu last modified on 04/19 2020

The objective of this tutorial is to introduce Point Cloud Library(PCL) and Iterative Closest Point(ICP).

Overview	1

Topics and Activities	2

Topic 1: PCLexample	2

Discussion and Checkpoint	2

Topic 2: Surface normal	3

Discussion.	3

Topic 3: Iterative Closest Point	3

Reference	3

# Overview

Estimated Time to Finish: 1.3 hours

After completing this tutorial you should

- be able to

  - run RealSense Camera

  - use ROS topics from RealSense Camera

  - use OpenCV

- know

  - how pointcloud is generated.

  - what is Morphological Image Processing

  - what is camera matrix

  - what is HSV color space

# Topics and Activities

## Topic 1: PCLexample

laptop$ cd 09-PCL_and_ICP/catkin_ws

laptop$ catkin_make

laptop$ source devel/setup.bash

(Make sure your RealSense Camera is connected to CP)

laptop$ roslaunch pcl_example topic1.launch

- Convert point cloud type from sensor_msgs::PointCloud2(ros) to pcl::PointCloud

- Information about pcl::PointCloudXYZRGB information

- How to remove the nan points from the original point cloud

### Discussion and Check point

- Please show the results

- How is the PCL pointcloud organized? Is there any relation between RGB image and pointcloud?

- What information can we get from a single point?

- Why does filtered point cloud have less points than original point cloud?

## Topic 2: Surface normal

laptop$ roslaunch pcl_example topic2.launch

- Using pcl to find point cloud surface normal

- pcl::search::KdTree - find the nearest point, pcl::NormalEstimation - find the normal

- visualization_msgs::Marker - ros tool helping you to visualize your work. There are many kind of markers. In this tutorial, we use visualization_msgs::Marker::LINE_LIST as the marker type.

- Method : use approximations to infer the surface normals from the point cloud.

  - for each point p in cloud P

1. get the nearest neighbors of p

2. compute the surface normal n of p（compute）

3. check if n is consistently oriented towards the viewpoint and flip otherwise

### Discussion and Check point

- Show the results of the topic 2

- Do you find any pointcloud pre-processing before calculating surface normal?

- If you increase the search range for surface normal, What will happen?

## Topic 3: Iterative Closet Point

laptop$ roslaunch pcl_example topic3.launch model:=[VIVA_model.ply file path]

cloud:=[VIVA_cloud.ply file path]

You can find this two files in /cpl_example/model/ folder.

Tip: click the file press ctrl+c, then move to terminal press shift+ctrl+v. it will paste the path of the file.

/camera/ini_guess & /camera/cloud

/camera/ini_guess & /camera/model

/camera/model & /camera/registered_cloud

- There are four functions in icp_example.cpp, addNormal, point_preprocess, initial_guess, point_2_plane_icp. Read the code to know how to do icp(with normal).

  - Preprocess pointcloud

    - Downsample

    - Denoise

  - Start from initial guess

    - Position: Centroid

    - Orientation: PCA

  - ICP

Method

    - For each point on M, find closest point on P

    - Find best transform for this corresponding

    - Transform M

Iter （criterion）

    - Number of iterations has reached the maximum user imposed number of iterations (setMaximumIterations)

    - The epsilon (difference) between the previous transformation and the current estimated transformation is smaller than an user imposed value (setTransformationEpsilon)

    - The sum of Euclidean squared errors is smaller than a user defined threshold (setEuclideanFitnessEpsilon)

Different kind of icp

    - pcl::GeneralizedIterativeClosestPoint< PointSource, PointTarget > Class Template Reference

    - pcl::IterativeClosestPoint< PointSource, PointTarget, Scalar > Class Template Reference

    - pcl::IterativeClosestPointWithNormals< PointSource, PointTarget, Scalar > Class Template Reference

    - pcl::IterativeClosestPointNonLinear< PointSource, PointTarget, Scalar > Class Template Reference

    - pcl::JointIterativeClosestPoint< PointSource, PointTarget, Scalar > Class Template Reference

    - pcl::registration::IncrementalICP< PointT, Scalar > Class Template Reference

### Discussion and Check point

There are other object models and corresponding object point cloud. Please change the model and corresponding cloud and observe the result.

- Show the result of topic 3

- Is it essential to do initial guess for ICP? If not, what will happen? Please show me the registration result that doesn’t execute initial guess before ICP.

- What do we use for  ICP termination criterion?

- What’s the difference between point-to-plane ICP and point-to-point ICP? (optional)

# Reference

- http://wiki.ros.org/rviz/Tutorials/Markers%3A%20Points%20and%20Lines#Using_Points.2C_Line_Strips.2C_and_Line_Lists

- http://eigen.tuxfamily.org/dox/group__TutorialBlockOperations.html

- https://eigen.tuxfamily.org/dox/classEigen_1_1Quaternion.html

- http://mediatum.ub.tum.de/doc/800632/941254.pdf